# High-Performance Workstation Build Report

## Components & Cost Table

| Component | Model / Specs | Cost (CAD) | Notes |
| --- | --- | --- | --- |
| CPU | AMD Ryzen Threadripper 7960X (24C/48T, 5.3 GHz) | $2,050 | Exceptional multi-thread performance for heavy workloads. |
| CPU Cooler | Noctua NH-U14S TR4-SP3 | $150 | Quiet, efficient cooling for Threadripper CPUs. |
| Motherboard | ASUS Pro WS WRX80E-SAGE SE WIFI | $1,000 | Workstation-grade, ECC memory support, many PCIe lanes. |
| RAM | 128GB DDR5 ECC Registered (4×32GB, 5200MHz) | $750 | Large capacity for virtualization and rendering. |
| GPU | NVIDIA RTX 4080 Super 16GB | $1,800 | High-end GPU for AI, rendering, and 4K editing. |
| Storage (Primary) | Samsung 990 Pro 2TB NVMe SSD | $300 | Ultra-fast boot and project load times. |
| Storage (Secondary) | Seagate IronWolf Pro 4TB HDD | $150 | Reliable large-capacity storage for assets. |
| Case | Fractal Design Define 7 XL | $300 | Spacious, silent, and excellent airflow. |
| PSU | Corsair AX1600i 1600W Platinum | $600 | High efficiency, future-proof power supply. |
| Warranty | 3-Year Extended Coverage | $200 | Covers parts & labor beyond manufacturer warranty. |
|  | Total (incl. 13% tax): | $5,980 |  |

Rationale & Tradeoffs:

Rationale:
This build prioritizes multi-threaded performance, GPU compute capability, and long-term reliability. The AMD Ryzen Threadripper 7960X provides unmatched rendering and parallel processing power for professional workloads such as 3D modeling, scientific computing, video production, and machine learning. The ASUS workstation motherboard ensures stability, ECC memory support, and scalability for additional GPUs or storage. The NVIDIA RTX 4080 Super was selected as a cost-balanced high-performance GPU capable of handling 4K video editing, real-time ray tracing, AI workflows, and large 3D scenes efficiently.

Storage was split into an ultra-fast 2TB NVMe SSD for OS, applications, and active projects, plus a 4TB enterprise-grade HDD for long-term storage. The Fractal Design Define 7 XL case and high-efficiency power supply ensure quiet operation, thermal headroom, and future upgrade capacity.

Tradeoffs Made:
- Downgraded from RTX 4090 to RTX 4080 Super to remain within budget while retaining strong GPU performance.
- Reduced HDD storage from 8TB to 4TB to allocate budget toward ECC RAM.
- Chose air cooling over AIO liquid cooling for lower maintenance and higher long-term reliability.
This configuration is balanced for CPU-heavy workloads without neglecting GPU compute capabilities, making it a future-proof investment for professional content creation and technical applications.

Screenshots: